SPEAR

Surveying and Planning through Electronic Applications and Referrals

# SCFF Instructions:  Exporting to DXF
AutoCAD
MicroStation CONNECT

# Exporting to DXF in AutoCAD, MicroStation CONNECT and LISCAD

## The information below is provided to assist surveyors in exporting an SCFF from AutoCAD and MicroStation CONNECT.  The information has been provided by the ePlan team and Surveyor-General Victoria teams, based on their experience using these applications.  If these instructions do not correlate with your own CAD package/customisation, please contact your vendor for more specific information.

| Exporting to DXF in AutoCAD | Option 1 - 'Save As', choose the File Type (AutoCAD 2010/LT2010 DXF (*.dxf)) - This will save the whole drawing which will remain open and the extension .dxf will be shown in the tab - Any changes done from this point will be to the DXF only and will need to be saved as a dwg if required when closing the file Option 2 - ‘Save As’, click on Tools on the top right corner and choose ‘Options’ - Click ‘DXF Options’, then ‘Select Objects’ and ‘OK’ - Choose the dxf version in the Files of type (AutoCAD 2010/LT2010 DXF (*.dxf)) and ‘Save’ - Select everything that you require and ‘Enter’ - The dxf will be saved in the folder and the opened drawing will remain a dwg |
| --- | --- |
| Exporting to DXF in MicroStation CONNECT | Option 1 - Export entire design file - Select File>Export>DWG - In the ‘Save As’ type field select - Autodesk® DXF Files (*.dxf) - At the bottom of the export dialog select ‘Options’. - Expand the directory ‘Basic’ and select the DWG Version - 2010/2011/2012. - Set any other required options and then select ‘OK’. - Choose the desired file location and enter the name of the export file in the File name field. - Select enter to save the export. Option 2 - Export selected/fenced elements - Place a fence using the fence tool, setting Fence Type to Block and Fence Mode to Inside. - Select File>Export>DWG - In the ‘Save As’ type field select - Autodesk® DXF Files (*.dxf) - At the bottom of the export dialog select ‘Options’. - On the General tab, expand the directory ‘Basic’ and select the DWG Version - 2010/2011/2012. - On the Filter tab, set Geometry to Fence and Fence Mode to Inside. - Set any other required options and then select ‘OK’. - Choose the desired file location and enter the name of the export file in the File name field. - Select ‘enter’ to save the export. NOTE: - Ensure cells are stored as type ‘point’ in cell/dgnlib library files. - Ensure cells are placed in the design file as ‘shared cells’ - Delete any existing ‘shared cell definitions’ from the design file before exporting to dxf. - The command/key in to be used is ‘delete scdefs all’. |