SPEAR

Surveying and Planning through Electronic Applications and Referrals

# SCFF Drawing Instructions
Section 32b
Plan to create an owners corporation (Existing Plan) |
Plan to create an owners corporation (New Plan)

# SCFF Drawing Instructions for Subdivision Act 1988 Primary Dealing Types

## Plans under:  Section 32b-Plan to create an owners corporation (existing plan), Section 32b-Plan to create an owners corporation (new plan)

| Drawing Instructions |  | Related Validation Rules |
| --- | --- | --- |
|  | Capture affected or new primary parcel(s) to be affected by the new Owners Corporation(s), using the following layer(s): - LOT-AFFECTED - RESERVE-AFFECTED - ROAD-AFFECTED - STAGE-LOT- AFFECTED - LOT-CREATED Note: For ‘Sec 32B-New Plan’, in addition to created lot, cancelled lot should also be captured in the LOT-CANCELLED layer. | Validation Type; Rule. SCFF; CAD-VR03-01 - CAD feature types. SCFF; CAD-VR03-02 - Polygon area. SCFF; CAD-VR04-02 - Geometry of common parcel boundaries. SCFF; CAD-VR04-03 - Geometry of overlapping boundaries. SCFF; CAD-VR04-04 - Self-intersecting parcels. SCFF; CAD-VR04-05 - Parcel vertices. SCFF; CAD-VR04-06 - Road connection to subject parcel. SCFF; CAD-VR04-08 - Boundary point uniqueness check. SCFF; CAD-VR04-09 - CAD feature uniqueness check. ePlan; VR003 - Multipart Parcel Completeness. ePlan; VR004 - Parcel Geometry Exists. ePlan; VR017 - Redundant Observations. ePlan; VR043 - Created Parcels for Cancelled Parcels. ePlan; VR063 - Parcel Area. ePlan; VR064 - Part Parcel Area Sum. ePlan; VR068 - Parcel Dimensions Exist. ePlan; VR069 - Parcel Geometry Closure. ePlan; VR071 - Parcel Observations Closure. ePlan; VR072 - Title Boundary Consistency. ePlan; VR096 - Observation Purpose and Point Type Consistency. ePlan; VR142 - Centre Point for Arc Observation. DCAS; VR501 - Arc Radius. DCAS; VR502 - Direction of Chord Bearing. DCAS; VR509 - Parcel Arc radius. DCAS; VR510 - Plan Misclosure. DCAS; VR511 - Parcel Arc Centre. DCAS; VR512 - Parcel Arc Centre Point Type. DCAS; VR513 - Parcel Direction of Chord Bearing. |
|  | Capture one label identifier per existing primary parcel within the parcel area according to the naming convention rules, using the following layer(s): - TEXT-LOT-ID - TEXT-RESERVE-ID - TEXT-ROAD-ID - TEXT-STAGE-LOT-ID | Validation Type; Rule. SCFF; CAD-VR03-01 - CAD feature types. SCFF; CAD-VR05-01 - Identifier label: parcel. SCFF; CAD-VR05-02 - Unique identifier label: parcel. SCFF; CAD-VR06-01 - Parcel naming convention. SCFF; CAD-VR06-02 - Unique parcel identifier label across plan. SCFF; CAD-VR06-03 - Minimum number of part parcels. ePlan; VR024 - Existing Parcel in VOTS. ePlan; VR039 - Extinguished Lot and Admin Area Consistency. ePlan; VR041 - Title Reference in VOTS. ePlan; VR042 - Parcel Name Format. ePlan; VR089 - Sequential Part Numbers. |
|  | Capture at least two title connections according to the SCFF Guidance Notes, using the following layer(s): - ROAD- EXISTING - CROWN-ALLOTMENT- EXISTING - CROWN-PORTION- EXISTING - LOT- EXISTING - RESERVE- EXISTING - COMMON-PROPERTY-EXISTING - STAGE-LOT- EXISTING | Validation Type; Rule. SCFF; CAD-VR03-01 - CAD feature types. SCFF; CAD-VR04-02 - Geometry of common parcel boundaries. SCFF; CAD-VR04-04 - Self-intersecting parcels. SCFF; CAD-VR04-05 - Parcel vertices. SCFF; CAD-VR04-06 - Road connection to subject parcel. SCFF; CAD-VR04-08 - Boundary point uniqueness check. SCFF; CAD-VR04-09 - CAD feature uniqueness check. ePlan; VR062 - Existing Crown Parcels. ePlan; VR073 - Title Connection. ePlan; VR101 - CoordGeom for Abuttals and Connections. DCAS; VR501 - Arc Radius. DCAS; VR502 - Direction of Chord Bearing. DCAS; VR509 - Parcel Arc radius. DCAS; VR510 - Plan Misclosure. DCAS; VR511 - Parcel Arc Centre. DCAS; VR512 - Parcel Arc Centre Point Type. DCAS; VR513 - Parcel Direction of Chord Bearing. |
|  | Capture existing easement parcels according to the SCFF Guidance Notes, using the following layer(s): - EASEMENT Make sure that you follow the rules for fixing easements to the title boundaries. | Validation Type; Rule. SCFF; CAD-VR03-01 - CAD feature types. SCFF; CAD-VR03-02 - Polygon area. SCFF; CAD-VR04-02 - Geometry of common parcel boundaries. SCFF; CAD-VR04-03 - Geometry of overlapping boundaries. SCFF; CAD-VR04-04 - Self-intersecting parcels. SCFF; CAD-VR04-05 - Parcel vertices. SCFF; CAD-VR04-08 - Boundary point uniqueness check. SCFF; CAD-VR04-09 - CAD feature uniqueness check. ePlan; VR003 - Multipart Parcel Completeness. ePlan; VR017 - Redundant Observations. ePlan; VR052 - Easement Fixing. ePlan; VR063 - Parcel Area. ePlan; VR064 - Part Parcel Area Sum. ePlan; VR068 - Parcel Dimensions Exist. ePlan; VR069 - Parcel Geometry Closure. ePlan; VR071 - Parcel Observations Closure. ePlan; VR096 - Observation Purpose and Point Type Consistency. ePlan; VR117 - Overlapping Parcels. ePlan; VR133 - Encumbering Road Easements. ePlan; VR134 - Easement Parcel Structure. ePlan; VR142 - Centre Point for Arc Observation. DCAS; VR501 - Arc Radius. DCAS; VR502 - Direction of Chord Bearing. DCAS; VR509 - Parcel Arc radius. DCAS; VR510 - Plan Misclosure. DCAS; VR511 - Parcel Arc Centre. DCAS; VR512 - Parcel Arc Centre Point Type. DCAS; VR513 - Parcel Direction of Chord Bearing. |
|  | Capture one label identifier per easement parcel within the parcel area according to the naming convention rules, using the following layers: - TEXT-EASEMENT-ID | Validation Type; Rule. SCFF; CAD-VR03-01 - CAD feature types. SCFF; CAD-VR05-01 - Identifier label: parcel. SCFF; CAD-VR05-02 - Unique identifier label: parcel. SCFF; CAD-VR05-03 - Unique identifier label: overlapping Easements. SCFF; CAD-VR06-01 - Parcel naming convention. SCFF; CAD-VR06-02 - Unique parcel identifier label across plan. SCFF; CAD-VR06-03 - Minimum number of part parcels. ePlan; VR042 - Parcel Name Format. ePlan; VR089 - Sequential Part Numbers. |
|  | If based on survey, capture survey details using the following layers: - POINT-CONTROL-PM - POINT-CONTROL-PCM - POINT-TRAVERSE - POINT-REFERENCE - LINE-TRAVERSE - LINE-RADIATION | Validation Type; Rule. SCFF; CAD-VR03-01 - CAD feature types. SCFF; CAD-VR04-01 - Traverse and radiation lines. SCFF; CAD-VR04-07 - Position of PMs, PCMs, Traverse Points and Reference Marks. SCFF; CAD-VR04-09 - CAD feature uniqueness check. ePlan; VR016 - Unconnected Points. ePlan; VR017 - Redundant Observations. ePlan; VR074 - Survey Marks Connection. ePlan; VR075 - Permanent Mark in SMES. ePlan; VR083 - PM in SMES. ePlan; VR088 - Non-boundary Observations. ePlan; VR096 - Observation Purpose and Point Type Consistency. ePlan; VR118 - Consistency of Datum. DCAS; VR503 - Connected Line. |
|  | If based on survey, capture one label identifier (9 figure number) per PM/PCM point according to the SCFF Guidance Notes, using the following layer: - TEXT-POINT-CONTROL-ID | Validation Type; Rule. SCFF; CAD-VR03-01 - CAD feature types. SCFF; CAD-VR05-01 - Identifier label: parcel. SCFF; CAD-VR05-05 - Identifier label: control points. SCFF; CAD-VR05-06 - Unique identifier label: control points. SCFF; CAD-VR06-01 - Parcel naming convention. SCFF; CAD-VR06-02 - Unique parcel identifier label across plan. ePlan; VR010 - PM and PCM Completeness. |